REVIEW OF WARPING THE LOOM

HOMEWORK for the Intermediate Class Lace — Table Runner

Winding the warp:

- Clamp the loom to a table with the back of the loom at the table’s edge.

- Place the reed/heddle in the neutral position. This project requires a 12-dent reed with a 12-inch weaving width.

- Clamp the warping peg to the other side of the table. You’ll need about 60-68 inches.

- Place your yarn on the floor near the loom. Use a bag, bowl or cone stand.

- Pull the back rod up and over the top of the warp beam.

- Find the center of the tie-on rod and measure half the project width to the right (The project is 12 inches wide, so measure 6 inches to the right of center). Tie the yarn there.

- From the center of the reed/heddle, measure half the project width to the right (6 inches). Mark it with a thread.

- Insert the hook and pull the yarn through that marked slot and circle around the warping peg. Use a little tension but don’t pull it too tight!

- Pull the yarn from the ball/cone through the next slot by going over the top of the tie-on rod. Loop it around the warping peg.

- Pull the next strand of yarn from the ball/cone and underneath the rod, through the slot, and around the peg.

- Continue threading slots, alternating over then under the tie-on rod.

- Once you’ve threading 6”, cut the yarn long enough so you can tie it to the tie-on rod. This will be one chain.

- Tie a piece of scrap yarn —a choke tie— tightly around the warp threads near the peg.

- Slip the yarn off the peg and cut it where it looped around the peg.

- Move the peg a bit to the left and repeat the steps above to make a second warp chain.

Threading the heddle/reed:

- Carefully slip one strand out of the slot and thread it through the hole just to the right. Repeat across the heddle. You should end up with 12 inches threaded, one thread in every slot, one thread in every hole.

- When finished, center the heddle/reed. Also center the tie-on rods.

Continued on page 2.

Rolling on:

- Ask a friend to hold the warp at the choke tie and apply some tension. If working alone, set something heavy near the choke tie (a few books). Or hang a small weight off the end of the table.

- Turn the crank on the back beam.

- When the yarn makes one full rotation around the beam, begin inserting warp sticks or heavy paper. Repeat, inserting 1-2 sticks per rotation.  Heavy paper should self-feed.

Tying on:

- Bring the front rod up and over the front beam.

- Starting just right of center, grasp about an inch of threads and divide in half.

- Pull the halves (one in each hand) over, then under the rod and then tie the ends over the 1” section like tying your shoes but with one extra throw.

- Repeat with the 1” group just to the left of center. Tighten the gear one click. Work across the warp, alternating groups to the left and right.

- Remove most of the slack that’s left with the ratchet gears.

- Next, slide the ends of the first group toward the reed/heddle, removing any additional slack. Use your thumbs. Pull the ends sideways and back toward yourself.  The 1” group of yarn should feel firm but not super tight. Repeat across the warp.

- Once the groups have even tension, tie each in a knot.