Student Handout for

Intermediate Weaving — Table Runner

Preparing to weave:

- Open the Up shed and insert a length of waste yarn, but don’t beat.

- Open the Down shed, insert a length of waste yarn near the first yarn, but don’t beat.

- Open the Up shed again, insert a third length of yarn. Now beat / pull all three picks at once until they’re about 2” from the tie-on rod. This spreads the warp threads evenly and moves any looseness from the weaving area. This is the only time you’ll beat 3 together.

- Winding the shuttle: Wind a few passes of weft yarn around the middle of the shuttle passing from one notch to the other. Then turn the shuttle so the thin edge is up and wind the weft yarn in a figure eight. The yarn will protrude off the “side” of the shuttle. Turn the shuttle to the other thin edge and repeat. This method prevents the shuttle from catching in the shed.

- As you pass the shuttle, do not allow it to pull in the outer-most threads, called the selvedges. Pinch the fabric to hold the edge steady as you gently pull the shuttle through the shed and snug up the yarn. Other options include temples or stretchers.

- Lay the weft yarn at an angle (a little less than a 45-degree angle). This is so important!

- Advance the warp regularly so you are weaving in the loom’s “sweet spot”.

- Weave 1-2” headers to check that the fabric is working (no threading mistakes). Also, evaluate the strength of your beat— that picks per inch (PPI) are correct. For this project it’s 12 ppi.

- Before you start weaving the actual the table runner, decide what kind of finished edges you’d like: hems or fringe. For hems, consider using a weft thread that’s half as thick as the rest of the project such as 8/2 cotton. Leave a 3” tail off the side. On your second pick, tuck that tail into the second shed. Weave 1.5-2 inches of hem.

If you prefer the hand-hem-stitching, follow these notes:

- On the first pick (pass of the shuttle) of your project, leave a tail of weft yarn hanging off to the side. This should be 3-4 times the width of your project. If right-handed, hang it off the right side, if left-handed, the left.

- After you have woven an inch of the table runner, put down the shuttle and thread the tail of yarn into a tapestry needle.

- Make a half-hitch knot around the right-most warp thread, then select a small group of threads. For this project it will be 4 ends.

- Pass the needle behind 4 warp threads and bring the needle forward again.

- Pass the needle around the back of the same 4 threads, and while back there, insert the needle into the fabric, two rows up. Snug up the yarn into a half hitch.

- Continue across the warp. Make an additional half hitch around the final thread on the left, then weave the tail into the fabric 1-2 inches.

Weaving the table runner

- After the hem or hemstitching there should be 1 inch of plain weave (heddle up, shuttle / heddle down, shuttle). End with a pick on the UP heddle.

- Place the heddle/reed in the down position. We’re only picking up threads located in slots. You can lay a sheet of paper inside the shed to make things easier to see.  Insert a pick-up stick, bevel down (if it has a bevel) in FRONT of the heddle reed …

Slide under 15 threads

(Over 1, under 1) x 7

Over 1, under 12

(Over 1, under 1) x 7

Over 1, under 15

Stand the stick on its edge up against the heddle/reed. Notice the shed that’s created behind the heddle/reed.  Insert a temporary pick-up stick or slat of wood in back and remove the first that was in front of the heddle/reed. Then replace temporary slat with your best pick-up stick.

- Next start the pattern. This is a 4-pick repeat:

- Put the heddle/reed in neutral.

Turn the stick on-end. Pass the shuttle.

Lay the stick flat and slide to the rear (it remains in the warp!)

Move the heddle UP, pass the shuttle.

Down, pass the shuttle

UP pass the shuttle.

- This is a 4-pick repeat that will continue until you nearly reach the end of your warp.

- Advance the warp often: Put the heddle/reed in neutral, turn the back gear away from you, lift the ratchet, then wind the fabric toward yourself.

- Continue the pattern as long as you like, just leave enough warp so you can finish the table runner with an inch of plainweave plus hems or fringe (roughly 3“ total).

- If you hemstitched at the start of the project, repeat the process at the end.

- If you’re weaving hems, repeat them at the end. Then weave some waste yarn to stabilize it for when you cut off.

- To cut a project from the loom, loosen the tension slightly. Don’t cut the Texsolv cords.

Finishing

- For rolled hems (rather than hemstitching / fringe) sew a straight line across the first pick of weaving and across the last pick of weaving. Catch all the warp threads.

- Wash gently in cold water. Hang dry. DON’T wring it or use the dryer. Block by hand.

- Now iron, roll and pin the hems, and sew the long side, plus both teeny tiny short sides.

- For hemstitching fringe, hang the fringe off the edge of a table with a weight on top of the fabric. Hand wash cold, block and dry flat. This weave wants to shrink!!

Kitchen towels:

If you have a weaving width of 20” you can also make kitchen towels with this lace effect.

For two towels, you will need a warp 3 yards long with a total of 240 threads. This means a total of two half-pound cones of 8/4 cotton yarn: one half-pound cone for the warp, and a second half-pound cone for the weft. You might wish to try the brand called Maurice Brassards 8/4 cotton. It’s not as stiff as 8/4 cotton rug warp. You might also wish to purchase a cone of 8/2 (Eight-two) cotton as well in a matching color for hems.

Warp in 2 chains … For direct warping, center the peg to one half of the loom, wind 120 threads (10 inches from center in the reed). Then move the peg, centering to the other half of the loom and direct warp the other 120 threads (10 inches). You may also use a warping board for indirect warping which may produce a better result.

Weave about 2 “ of plainweave for hems (try a finer yarn like 8/2 cotton).

Weave an additional inch of plainweave as the start of the towel ending with the last pick on an UP heddle. Then move the heddle/reed in the DOWN position insert the pick-up stick like so:

Slide under 11 threads

*(Over 1, under 1) x 5

Over 1, under 11

Repeat from the * three more times, then

(Over 1, under 1) x 5

Over 1, under 10

Then begin weaving the lace pattern:

Put the heddle/reed in neutral. Turn the stick on end. Pass the shuttle.

Lay the stick flat (it remains in the warp!)

Move the heddle UP, pass the shuttle.

Down, pass the shuttle

UP pass the shuttle.

After weaving about 28” of lace pattern it’s time to finish up the first towel… weave 1 inch of plainweave, and then for the hems, 2 inches of plainweave in a finer thread like 8/2 cotton. Place a few strands of waste yarn to divide the two towels (so it’s easy to cut them apart after washing), and repeat these instructions for the next towel.

Once you cut the towels from the loom take them to the sewing machine, run a straight line of stitching across each of the four hem edges. This will keep them from unraveling when you wet finish them, cut them apart, and before their final hemming.

After gently washing, hem and iron. They make lovely gifts for yourself or friends!